# BMP280 Linux Kernel I2C Driver

## 1. Purpose

This document provides a strict execution-based guide for building, loading, validating, and debugging the BMP280 Linux kernel I2C driver.

For every step, the following are clearly specified:

- Exact command to run

- Expected output (including cases where no output is success)

- Meaning if the expected output does NOT occur

- Corrective action and corrected command

This document is designed so that no kernel background knowledge is required. If followed line-by-line, the driver will work.

## 2. I2C Interface Enable Verification

### Command

ls /dev/i2c-1

### Expected Output

/dev/i2c-1

### If Output Is Missing

I2C is disabled in firmware.

### Rectification

sudo raspi-config
# Interface Options → I2C → Enable
sudo reboot

Re‑verify using the same command.

## 3. Hardware‑Level Sensor Detection (Mandatory)

### Command

sudo i2cdetect -y 1

### Expected Output

0 1 2 3 4 5 6 7 8 9 a b c d e f
70: -- -- -- -- -- -- 76 --

### Interpretation

0x76 confirms BMP280 presence on I2C bus 1.

### If Address 0x76 Is Not Visible

| Possible Cause | Corrective Action |
| --- | --- |
| Incorrect wiring | Verify SDA → GPIO2, SCL → GPIO3 |
| Wrong voltage | BMP280 requires 3.3V only |
| Faulty module | Replace sensor |

Do not proceed until this step passes.

4.Device Tree

/dts-v1/;
/plugin/;
/ {
    compatible = "brcm,bcm2835";
fragment@0 {
        target-path = "/soc/i2c@7e804000";
        __overlay__ {
            #address-cells = <1>;
            #size-cells = <0>;
            status = "okay";
 bmp280@76 {
                compatible = "bosch,bmp280";
                reg = <0x76>;
                status = "okay";
            };
        };
    };
};

## 5. Device Tree Overlay Compilation

### Command

dtc -@ -I dts -O dtb -o bmp280.dtbo bmp280.dts

### Expected Output

No output (successful compilation).

### If Compilation Fails

Example error:

Error: target-path not found

### Rectification

Ensure correct Raspberry Pi 4 I2C controller path in DTS:

target-path = "/soc/i2c@7e804000";

Recompile using the same command.

## 6. Overlay Installation

### Command

sudo cp bmp280.dtbo /boot/overlays/

### Expected Output

No output.

### Verification

ls /boot/overlays | grep bmp280

Expected:

bmp280.dtbo

### If Missing

Re-copy with explicit filename:

sudo cp bmp280.dtbo /boot/overlays/bmp280.dtbo

## 7. Overlay Enablement

### Command

sudo nano /boot/config.txt

Add line:

dtoverlay=bmp280

### Apply

sudo reboot

## 8. Device Tree → I2C Binding Verification

### Command

ls /sys/bus/i2c/devices/

### Expected Output

1-0076

Check modalias

cat /sys/bus/i2c/devices/1-0076/modalias

Expected output

i2c:bmp280

Check driver registered

ls /sys/bus/i2c/drivers/

Expected output

bmp280

Check device ↔ driver binding

ls -l /sys/bus/i2c/drivers/bmp280/

Expected output

1-0076 -> ../../devices/platform/.../1-0076

Manual bind (if needed)

echo 1-0076 > /sys/bus/i2c/drivers/bmp280/bind

Expected output

(no output)

### If Not Present

| Cause | Fix |
| --- | --- |
| Overlay not loaded | Verify config.txt |
| Wrong bus | Ensure i2c-1 |
| DTS error | Recompile overlay |

Debug command:

dmesg | grep bmp280

9.Kernel Driver

// SPDX-License-Identifier: GPL-2.0
#include <linux/module.h>
#include <linux/kernel.h>
#include <linux/i2c.h>
#include <linux/delay.h>
#include <linux/fs.h>
#include <linux/uaccess.h>
#include <linux/cdev.h>
#include <linux/device.h>
#include <linux/slab.h>
#include <linux/workqueue.h>
#include <linux/mutex.h>
#include <linux/timekeeping.h>
#include <linux/file.h>
#include <linux/err.h>
#include <linux/string.h>
#define BMP280_CHIP_ID_REG 0xD0
#define BMP280_CHIP_ID     0x58
#define BMP280_CTRL_MEAS   0xF4
#define BMP280_CONFIG      0xF5
#define BMP280_PRESS_MSB   0xF7
#define DEVICE_NAME        "bmpdr"
#define BMP280_POLL_MS     1000
#define BMPDR_LOG_FILE     "/var/log/bmpdr.log"
struct bmp280_calib {
    u16 dig_T1;
    s16 dig_T2;
    s16 dig_T3;
    u16 dig_P1;
    s16 dig_P2;
    s16 dig_P3;
    s16 dig_P4;
    s16 dig_P5;
    s16 dig_P6;
    s16 dig_P7;
    s16 dig_P8;
    s16 dig_P9;
};
struct bmpdr_data {
    struct i2c_client *client;
    struct bmp280_calib calib;
    int32_t t_fine;
    char buffer[64];           // latest sensor reading
    struct cdev cdev;
    struct delayed_work dwork; // periodic read
    struct mutex lock;         // protect i2c & buffer
};
static dev_t bmpdr_dev;
static struct class *bmpdr_class;
/* ---------------- Log Write Function ---------------- */
static void bmpdr_write_log(struct bmpdr_data *d)
{
    struct file *filp;
    char logbuf[128];
    struct timespec64 ts;

ktime_get_real_ts64(&ts);
snprintf(logbuf, sizeof(logbuf),
             "[%lld] %s\n",
             (long long)ts.tv_sec,
             d->buffer);
filp = filp_open(BMPDR_LOG_FILE, O_WRONLY | O_CREAT | O_APPEND, 0644);
    if (IS_ERR(filp))
        return;
 kernel_write(filp, logbuf, strlen(logbuf), &filp->f_pos);
    filp_close(filp, NULL);
}
/* ---------------- Calibration Read ---------------- */
static int bmpdr_read_calibration(struct i2c_client *client,
                                  struct bmp280_calib *c)
{
    u8 buf[24];
    int ret, i;

    for (i = 0; i < 3; i++) {
        ret = i2c_smbus_read_i2c_block_data(client, 0x88, 24, buf);
        if (ret >= 0)
            break;
        msleep(10);
    }
    if (ret < 0)
        return ret;
c->dig_T1 = (buf[1] << 8) | buf[0];
    c->dig_T2 = (s16)((buf[3] << 8) | buf[2]);
    c->dig_T3 = (s16)((buf[5] << 8) | buf[4]);
    c->dig_P1 = (buf[7] << 8) | buf[6];
    c->dig_P2 = (s16)((buf[9] << 8) | buf[8]);
    c->dig_P3 = (s16)((buf[11] << 8) | buf[10]);
    c->dig_P4 = (s16)((buf[13] << 8) | buf[12]);
    c->dig_P5 = (s16)((buf[15] << 8) | buf[14]);
    c->dig_P6 = (s16)((buf[17] << 8) | buf[16]);
    c->dig_P7 = (s16)((buf[19] << 8) | buf[18]);
    c->dig_P8 = (s16)((buf[21] << 8) | buf[20]);
    c->dig_P9 = (s16)((buf[23] << 8) | buf[22]);
 return 0;
}
/* ---------------- Compensation ---------------- */
static int32_t bmpdr_compensate_temp(struct bmpdr_data *d, int32_t adc_T)
{
    int32_t var1, var2;
    var1 = ((((adc_T >> 3) - ((int32_t)d->calib.dig_T1 << 1))) *
            ((int32_t)d->calib.dig_T2)) >> 11;
    var2 = (((((adc_T >> 4) - ((int32_t)d->calib.dig_T1)) *
              ((adc_T >> 4) - ((int32_t)d->calib.dig_T1))) >> 12) *
            ((int32_t)d->calib.dig_T3)) >> 14;
    d->t_fine = var1 + var2;
    return (d->t_fine * 5 + 128) >> 8;
}

static uint32_t bmpdr_compensate_pressure(struct bmpdr_data *d, int32_t adc_P)
{
    int64_t var1, var2, p;
    var1 = (int64_t)d->t_fine - 128000;
    var2 = var1 * var1 * d->calib.dig_P6;
    var2 += (var1 * d->calib.dig_P5) << 17;
    var2 += ((int64_t)d->calib.dig_P4) << 35;
    var1 = ((var1 * var1 * d->calib.dig_P3) >> 8) +
           ((var1 * d->calib.dig_P2) << 12);
    var1 = (((((int64_t)1) << 47) + var1) * d->calib.dig_P1) >> 33;
    if (var1 == 0)
        return 0;
    p = 1048576 - adc_P;
    p = (((p << 31) - var2) * 3125) / var1;
    var1 = (d->calib.dig_P9 * (p >> 13) * (p >> 13)) >> 25;
    var2 = (d->calib.dig_P8 * p) >> 19;
    p = ((p + var1 + var2) >> 8) + ((int64_t)d->calib.dig_P7 << 4);
    return (uint32_t)p;
}

/* ---------------- Sensor Read ---------------- */
static int bmpdr_read_sensor(struct bmpdr_data *d)
{
    u8 buf[6];
    int32_t adc_T, adc_P, temp;
    uint32_t press;
    int ret, i;
    u8 id;
    mutex_lock(&d->lock);
 for (i = 0; i < 3; i++) {
        id = i2c_smbus_read_byte_data(d->client, BMP280_CHIP_ID_REG);
        if (id == BMP280_CHIP_ID)
            break;
        msleep(20);
    }
    if (id != BMP280_CHIP_ID) {
        mutex_unlock(&d->lock);
        return -ENODEV;
    }

for (i = 0; i < 3; i++) {
        ret = i2c_smbus_read_i2c_block_data(d->client, BMP280_PRESS_MSB, 6, buf);
        if (ret >= 0)
            break;
        msleep(20);
    }
    if (ret < 0) {
        mutex_unlock(&d->lock);
        return -EIO;
    }
 adc_P = (buf[0] << 12) | (buf[1] << 4) | (buf[2] >> 4);
    adc_T = (buf[3] << 12) | (buf[4] << 4) | (buf[5] >> 4);
temp = bmpdr_compensate_temp(d, adc_T);
    press = bmpdr_compensate_pressure(d, adc_P);
snprintf(d->buffer, sizeof(d->buffer),
             "Temperature: %d.%02d C, Pressure: %d.%02d hPa",
             temp / 100, temp % 100,
             press / 25600, (press % 25600) / 256);
/* Write to log file */
    bmpdr_write_log(d);
mutex_unlock(&d->lock);
    return 0;
}
/* ---------------- Workqueue ---------------- */
static void bmpdr_work_func(struct work_struct *work)
{
    struct bmpdr_data *d = container_of(work, struct bmpdr_data, dwork.work);
    bmpdr_read_sensor(d);
    schedule_delayed_work(&d->dwork, msecs_to_jiffies(BMP280_POLL_MS));
}
/* ---------------- Char Device ---------------- */
static ssize_t bmpdr_read(struct file *f, char __user *buf,
                          size_t cnt, loff_t *ppos)
{
    struct bmpdr_data *d = f->private_data;
    return simple_read_from_buffer(buf, cnt, ppos, d->buffer, strlen(d->buffer));
}
static int bmpdr_open(struct inode *inode, struct file *f)
{
    f->private_data = container_of(inode->i_cdev, struct bmpdr_data, cdev);
    return 0;
}
static const struct file_operations bmpdr_fops = {
    .owner = THIS_MODULE,
    .read  = bmpdr_read,
    .open  = bmpdr_open,
};

/* ---------------- Probe ---------------- */
static int bmpdr_probe(struct i2c_client *client)
{
    struct bmpdr_data *d;
    int ret;
  d = devm_kzalloc(&client->dev, sizeof(*d), GFP_KERNEL);
    if (!d)
        return -ENOMEM;
d->client = client;
    mutex_init(&d->lock);
    i2c_set_clientdata(client, d);
  ret = bmpdr_read_calibration(client, &d->calib);
    if (ret < 0)
        return ret;
  /* Sensor init */
    i2c_smbus_write_byte_data(client, BMP280_CTRL_MEAS, 0x27);
    i2c_smbus_write_byte_data(client, BMP280_CONFIG, 0xA0);
    msleep(10);
/* Char device */
    ret = alloc_chrdev_region(&bmpdr_dev, 0, 1, DEVICE_NAME);
    if (ret < 0)
        return ret;
bmpdr_class = class_create(DEVICE_NAME);
    if (IS_ERR(bmpdr_class)) {
        unregister_chrdev_region(bmpdr_dev, 1);
        return PTR_ERR(bmpdr_class);
    }
 cdev_init(&d->cdev, &bmpdr_fops);
    cdev_add(&d->cdev, bmpdr_dev, 1);
    device_create(bmpdr_class, NULL, bmpdr_dev, NULL, DEVICE_NAME);
/* Start periodic work */
    INIT_DELAYED_WORK(&d->dwork, bmpdr_work_func);
    schedule_delayed_work(&d->dwork, msecs_to_jiffies(BMP280_POLL_MS));
/* Probe log message */
    dev_info(&client->dev, "bmpdr probe successful\n");
return 0;
}
static void bmpdr_remove(struct i2c_client *client)
{
    struct bmpdr_data *d = i2c_get_clientdata(client);
    cancel_delayed_work_sync(&d->dwork);
    device_destroy(bmpdr_class, bmpdr_dev);
    cdev_del(&d->cdev);
    class_destroy(bmpdr_class);
    unregister_chrdev_region(bmpdr_dev, 1);
}
static const struct of_device_id bmpdr_of_match[] = {
    { .compatible = "bosch,bmp280" },
    { }
};
MODULE_DEVICE_TABLE(of, bmpdr_of_match);
static const struct i2c_device_id bmpdr_id[] = {
    { "bmpdr", 0 },
    { }
};
MODULE_DEVICE_TABLE(i2c, bmpdr_id);
static struct i2c_driver bmpdr_driver = {
    .driver = {
        .name = "bmpdr",
        .of_match_table = bmpdr_of_match,
    },
    .probe  = bmpdr_probe,
    .remove = bmpdr_remove,
    .id_table = bmpdr_id,
};
static int __init bmpdr_init(void) { return i2c_add_driver(&bmpdr_driver); }
static void __exit bmpdr_exit(void) { i2c_del_driver(&bmpdr_driver); }
module_init(bmpdr_init);
module_exit(bmpdr_exit);
MODULE_LICENSE("GPL");
MODULE_AUTHOR("Srikanth Reddy");
MODULE_DESCRIPTION("BMP280 I2C Driver: char device + periodic file logging (no dmesg readings)");

9.Create Makefile

obj-m += driver_file name .o

KDIR ?= /lib/modules/$(shell uname -r)/build

PWD  := $(shell pwd)

all:

make -C $(KDIR) M=$(PWD) modules

clean:

make -C $(KDIR) M=$(PWD) clean

## 10. Kernel Driver Compilation

### Command

make

### Expected Output

CC [M]  bmpdr.o
LD [M]  bmpdr.ko

### If Build Fails

| Error | Fix |
| --- | --- |
| Missing Makefile | Ensure correct directory |
| Header errors | Install kernel headers |

## 11. Kernel Module Insertion

### Command

sudo insmod bmpdr.ko

### Expected Output

No output.

### If Error Appears

Invalid module format

### Rectification

make clean
make
sudo insmod bmpdr.ko

## 12. Probe Function Confirmation

### Command

dmesg | tail -20

### Expected Output

bmpdr: probe successful
bmpdr: BMP280 detected (chip id 0x58)

### If Probe Did Not Execute

| Cause | Fix |
| --- | --- |
| DT mismatch | Verify compatible string |
| Wrong address | Must be 0x76 |
| Driver not registered | Ensure i2c_add_driver() |

## 13. Character Device Verification

### Command

ls -l /dev/bmpdr

### Expected Output

crw------- 1 root root ... /dev/bmpdr

### If Missing

Check kernel log:

dmesg | grep bmpdr

## 14. Sensor Data Read Test

### Command

cat /dev/bmpdr

### Expected Output

Temperature: XX.XX C, Pressure: XXXX.XX hPa

### If Output Is Empty or Blocks

| Cause | Fix |
| --- | --- |
| Workqueue not scheduled | Check delayed_work |
| Mutex issue | Validate locking |
| I2C read error | Recheck wiring |

## 15. Periodic Logging Verification

### Command

sudo cat /var/log/bmpdr.log

### Expected Output

[epoch] Temperature: ...
[epoch] Temperature: ...

### If File Missing

| Cause | Fix |
| --- | --- |
| File not created | Use O_CREAT flag |
| Permission issue | Run as root |

Live monitoring:

tail -f /var/log/bmpdr.log

## 16. Module Removal Verification

### Command

sudo rmmod bmpdr

### Expected Output

No output.

### Verify Cleanup

ls /dev/bmpdr

Expected:

No such file or directory

## 17. Final Validation Checklist

- I2C address 0x76 visible

- Device Tree overlay loaded

- Probe executed successfully

- Character device created

- Sensor data readable

- Periodic logging functional

- Clean module removal

## 18. Conclusion

This document represents a complete execution‑level validation manual for the BMP280 Linux kernel I2C driver. Each step is deterministic, verifiable, and recoverable, making it suitable for project submission, production bring‑up, and technical interviews.